FTDI CHIP

Isolation 5000VDC

Industrial specification

USB to serial converters from Single chip USB to RS232

USB 1.0/1.1 compliant, USB 2.0 Full Speed compatible

12 Mbps USB data rate

Transmit and receive LED drive .

Serial Interface 1 Port

15 KV ESD protection for all serial ports

Connector: DB9 Male and MINI USB B type

Cable:USB Type A and MINI USB B Type

.Driver Support

Windows 98, 98SE, ME

Windows 2000

Windows XP and XP 64-bit

Windows Vista and Vista 64-bit

Windows Server 2003 and 2003 64-bit

Windows Server 2008 and 2008 64-bit

Windows 7 and 7 64-bit

Mac OS8/9,OS-X

Linux 2.4 and greater

Applications

.Notebooks and Industrial PCs peripherals devices

.Terminal Adapters and POS terminals

.Peripherals interface

.General-purpose RS-232 data link

Serial Converter

| Mode | Asynchronous serial communication |
| --- | --- |
| Speed | Up to 921.6kbps |
| Connector | DB9 Male |
| Signals | TXD,RXD,RTS,CTS,DTR,DSR,DCD ,RI ,GND |
| Protection | 15KV ESD Protection for all signals |
| Data Bits | 7,8 |
| Stop Bits | 1,2 |
| Flow Control | RTS/CTS,X-On/X-Off |
| Parity | None,Even,Odd,Space,Mark |
| IRQ/IO Address | Assigned by BIOS |
| FIFO | TX Buffer 128 bytes, RX Buffer 256 bytes |

Software

| Driver Support | Windows 98, 98SE, ME, 2000, Server 2003, XP ,Server 2008, XP 64-bit Windows Vista, Vista 64-bit,Server 2008 R2,Windows 7,Windows 7 X64 Mac OS 8/9, OS-X Linux 2.4 and greater |
| --- | --- |

Hardware

| Power Input | MINI USB B type connector |
| --- | --- |
| Power Consumption | 55mA max |
| LED | TXD,RXD |
| Dimensions | 42.5(W)x22.5(H)85(D)mm |
| Weight | 54g |

Connector DB9 Male 9PIN Define

| NO | DM9 Male |
| --- | --- |
| 1 | DCD |
| 2 | RXD |
| 3 | TXD |
| 4 | DTR |
| 5 | GND |
| 6 | DSR |
| 7 | RTS |
| 8 | CTS |
| 9 | RI |

Environmental

| Operating Temp | -30~75℃ |
| --- | --- |
| Storage Temp | -40~85℃ |
| Humidity | 5~95% Non-Condensing |
| Case | PC |

Approvals

CE ,FCC RoHs Compliant

Dimension

DIN rail Dimension