Specification

FTDI CHIP

Industrial specification

Isolation 3000VDC

USB to serial converters from Single chip USB to RS422/485

USB 1.0/1.1 compliant, USB 2.0 Full Speed compatible

12 Mbps USB data rate

Transmit and receive LED drive .

Built-in terminator 120 ohm for DIP switch setting

32 Device Connectivity for RS485 Max.

Serial Interface 1 Port

15 KV ESD protection for all serial ports and RS422/485 signals Surge Protection

Connector: Terminal Block 5P and MINI USB B type

Cable:USB Type A and MINI USB B Type

.Driver Support

Windows 98, 98SE, ME

Windows 2000

Windows XP and XP 64-bit

Windows Vista and Vista 64-bit

Windows Server 2003 and 2003 64-bit

Windows Server 2008 and 2008 64-bit

Windows 7 and 7 64-bit

Mac OS8/9,OS-X

Linux 2.4 and greater

Applications

.RS-422/485 communication

.Utility meters

.Industrial process control

.Building automation

.Industrial-controlled Local Area Network

.Industrial PCs and PC peripherals devices

Serial Converter

| Mode | Asynchronous serial communication |
| --- | --- |
| Speed | Up to 460.8kbps |
| Connector | Terminal Block 5P 5.08mm |
| Signals | TX+,TX-,RX+,RX- ,SGND |
| Protection | 15KV ESD Protection for all signals RS422/485 signals Surge Protection |
| Data Bits | 7,8 |
| Stop Bits | 1,2 |
| Flow Control | RTS/CTS,XON/XOFF |
| Parity | None,Even,Odd,Space,Mark |
| IRQ | Assigned by BIOS |
| FIFO | TX Buffer 128 bytes, RX Buffer 256 bytes |
| Switch | RS422 Term.Resistor setting |
|  | RS485 Term.Resistor setting |
|  | RS485 Echo/non-Echo selection |
|  | RS422/485 selection |

Software

| Driver Support | Windows 98, 98SE, ME, 2000, Server 2003, XP ,Server 2008, XP 64-bit Windows Vista, Vista 64-bit,Server 2008 R2,Windows 7,Windows 7 X64 Mac OS 8/9, OS-X Linux 2.4 and greater |
| --- | --- |

Hardware

| Power Input | MINI USB B type connector |
| --- | --- |
| Power Consumption | 54mA max |
| LED | TXD,RXD |
| Isolation | Optical 3000 VDC max |
| Dimensions | 42.5(W)x22.5(H)94.5(D)mm |
| Weight | 56g |

Terminal Block PIN Define

|  | RS422 | RS422Multi-Drop/RS485 4W | RS485 |
| --- | --- | --- | --- |
| 1 | TX+ | TX+ |  |
| 2 | TX- | TX+ |  |
| 3 | RX+ | RX+ | DATA+ |
| 4 | RX- | RX- | DATA- |
| 5 | SG | SG | SG |

DIP SW

| DIP SW SET | 1 | 2 | 3 | 4 |  |
| --- | --- | --- | --- | --- | --- |
| 0=OFF,1=ON | 0 | 0 | 0 | x | RS422 |
|  | 0 | 0 | 1 | x | RS422 |
|  | 1 | 0 | 1 | x | RS485 4W,RS422 Multi Drop |
|  | 0 | 1 | 0 | x | RS485 2W NO ECHO |
|  | 0 | 1 | 1 | x | RS485 2W ECHO |
|  | x | x | x | 1 | RS422/485 Term.Resistor ON |

Environmental

| Operating Temp | -30~75℃ |
| --- | --- |
| Storage Temp | -40~85℃ |
| Humidity | 5~95% Non-Condensing |
| Case | PC |

Approvals

CE ,FCC RoHs Compliant

Dimension

DIN rail Dimension

Driver installation guide for windows XP

STEP 1.       1port         2port         4port

STEP 2.

STEP 3.

SETP 4.

SETP 5.

SETP 6.

SETP 7.

SETP 8.    1port            2port           4port

SETP 9.

.

STEP 10..

SETP 11.

2port Loop STEP 9~11  two next

4port Loop STEP 9~11  four next

USB to 1port RS232/422/485

USB to 2port RS232

USB to 4port RS232

Driver installation guide for windows 7

Step 1

Step2  Hardware and Sound

Step3  Devices and Printers

Step 4 Other device

Step 5 Update Driver Software

Step 6

Step 7

Step 8

Step 9

Step 10 USB Serial Port

Step 11 Update Driver Software

Step 12

Step 13

Step 14

Step 15

Step 16