Specification

FTDI CHIP

Industrial specification

USB to serial converters from Single chip USB to RS232

USB 2.0 Full Speed ,USB 1.0/1.1 compliant

480 Mbps USB data rate

Transmit and receive LED drive .

Serial Interface 2 Port

15 KV ESD protection for all serial ports

Connector: DB9 Male and MINI USB B type

Cable:USB Type A and MINI USB B Type

.Driver Support

Windows 2000

Windows XP and XP 64-bit

Windows Vista and Vista 64-bit

Windows Server 2003 and 2003 64-bit

Windows Server 2008 and 2008 64-bit

Windows 7 and 7 64-bit

Mac OS-X

Linux 2.4 and greater

Applications

.Notebooks and Industrial PCs peripherals devices

.Terminal Adapters and POS terminals

.Peripherals interface

.General-purpose RS-232 data link

Serial Converter

| Mode | Asynchronous serial communication |
| --- | --- |
| Speed | Up to 921.6kbps |
| Connector | DB9 Male*2 |
| Signals | TXD,RXD,RTS,CTS,DTR,DSR,DCD ,RI ,GND |
| Protection | 15KV ESD Protection for all signals |
| Data Bits | 7,8 |
| Stop Bits | 1,2 |
| Flow Control | RTS/CTS, X-On/X-Off |
| Parity | None,Even,Odd,Space,Mark |
| IRQ/IO Address | Assigned by BIOS |
| FIFO | TX Buffer 4k bytes, RX Buffer 4k bytes per channel |

Software

| Driver Support | Windows 98, 98SE, ME, 2000, Server 2003, XP ,Server 2008, XP 64-bit Windows Vista, Vista 64-bit, Server 2008 R2,Windows 7,Windows 7 X64 Mac OS-X Linux 2.4 and greater |
| --- | --- |

Hardware

| Power Input | MINI USB B type connector |
| --- | --- |
| Power Consumption | 115mA max |
| LED | TXD1,RXD1,TXD2,RXD2 |
| Dimensions | 60.1(W)x22.5(H)80.5(D)mm |
| Weight | 65g |

Connector DB9 Male 9PIN Define

| NO | DM9 Male |
| --- | --- |
| 1 | DCD |
| 2 | RXD |
| 3 | TXD |
| 4 | DTR |
| 5 | GND |
| 6 | DSR |
| 7 | RTS |
| 8 | CTS |
| 9 | RI |

Environmental

| Operating Temp | -30~75℃ |
| --- | --- |
| Storage Temp | -40~85℃ |
| Humidity | 5~95% Non-Condensing |
| Case | PC |

Approvals

CE ,FCC RoHs Compliant

Dimension

DIN rail Dimension

Driver installation guide for windows XP

STEP 1.       1port         2port         4port

STEP 2.

STEP 3.

SETP 4.

SETP 5.

SETP 6.

SETP 7.

SETP 8.    1port            2port           4port

SETP 9.

.

STEP 10..

SETP 11.

2port Loop STEP 9~11  two next

4port Loop STEP 9~11  four next

USB to 1port RS232/422/485

USB to 2port RS232

USB to 4port RS232

Driver installation guide for windows 7

Step 1

Step2  Hardware and Sound

Step3  Devices and Printers

Step 4 Other device

Step 5 Update Driver Software

Step 6

Step 7

Step 8

Step 9

Step 10 USB Serial Port

Step 11 Update Driver Software

Step 12

Step 13

Step 14

Step 15

Step 16