SOHO KVM User Manual

14.01.3234

Specifications

- Operating temperature: 0 ~ 50℃

- Weight: 360g

Features

- Operate two PS/2 / USB computers with one keyboard, mouse and monitor.

- Simply plug the attached cables-nothing else required.

- Full keyboard and mouse emulation for error-free booting.

- Hot key commands and LED indicators for computer selection.

- No software and power required.

- Support video resolution up to 1920 × 1440.

- Compatible with all operating systems.

Installation

- Make sure that power to all the devices you will be connecting to have been turned off, and that all devices on the installation are properly grounded.

- Plug your keyboard, mouse and monitor into the unit’s console ports.

- Plug the device’s built-in cables into the keyboard, mouse and monitor ports of your computers. The keyboard and the mouse plugs are color-coded and marked with an indicator icon.

- This completes the installation. Power on your computers and you are ready to go.

Operation

Toggle the KVM control between the computers by tapping the CTRL key twice.